EXP – 6

clear; clc; close all;

Fs = 8000;

fc = 1500;

wc = fc / (Fs/2);       % Normalized cutoff frequency

N = 50;                 % Filter order

% Window Functions

w_rect  = rectwin(N+1);

w_hamm  = hamming(N+1);

w_hann  = hann(N+1);

w_black = blackman(N+1);

% FIR Low-Pass Filter Design using fir1

b_rect  = fir1(N, wc, w_rect);

b_hamm  = fir1(N, wc, w_hamm);

b_hann  = fir1(N, wc, w_hann);

b_black = fir1(N, wc, w_black);

% Frequency Response Calculation

[H_rect, f] = freqz(b_rect, 1, 1024, Fs);

[H_hamm, ~] = freqz(b_hamm, 1, 1024, Fs);

[H_hann, ~] = freqz(b_hann, 1, 1024, Fs);

[H_black, ~] = freqz(b_black, 1, 1024, Fs);

% Display Numerical Outputs

fprintf('=== EXPERIMENT 4: FIR WINDOWING TECHNIQUES ===\n\n');

fprintf('Filter Order (N): %d\n', N);

fprintf('Cutoff Frequency: %d Hz\n\n', fc);

fprintf('--- RECTANGULAR WINDOW COEFFICIENTS (First 5) ---\n');

disp(b_rect(1:5));

fprintf('--- HAMMING WINDOW COEFFICIENTS (First 5) ---\n');

disp(b_hamm(1:5));

fprintf('--- HANN WINDOW COEFFICIENTS (First 5) ---\n');

disp(b_hann(1:5));

fprintf('--- BLACKMAN WINDOW COEFFICIENTS (First 5) ---\n');

disp(b_black(1:5));

% Convert Magnitude to dB safely

mag_rect  = 20*log10(max(abs(H_rect), 1e-10));

mag_hamm  = 20*log10(max(abs(H_hamm), 1e-10));

mag_hann  = 20*log10(max(abs(H_hann), 1e-10));

mag_black = 20*log10(max(abs(H_black), 1e-10));

% Plot Frequency Responses

figure('Name', 'FIR Windowing Techniques Comparison');

subplot(2,1,1);

plot(f, mag_rect, 'k', 'LineWidth', 1.2);

hold on;

plot(f, mag_hamm, 'b', 'LineWidth', 1.5);

plot(f, mag_hann, 'g', 'LineWidth', 1.5);

plot(f, mag_black, 'r', 'LineWidth', 1.5);

grid on;

legend('Rectangular', 'Hamming', 'Hann', 'Blackman', ...

'Location', 'best');

title('FIR Low-Pass Filter Magnitude Response (N = 50)');

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

xlim([0 Fs/2]);

ylim([-100 10]);

% Phase Response using Hamming Window

subplot(2,1,2);

plot(f, unwrap(angle(H_hamm))*180/pi, ...

'b', 'LineWidth', 1.5);

grid on;

title('Phase Response (Hamming Window)');

xlabel('Frequency (Hz)');

ylabel('Phase (degrees)');

xlim([0 Fs/2]);

EXP -7

clear; clc; close all;

Fs = 10000;

fp = 1500;

fs = 2000;

N = 40;                 % Filter order

% 1. Parks-McClellan (Equiripple) Design

f_bands = [0 fp fs Fs/2] / (Fs/2);

a_bands = [1 1 0 0];

b_pm = firpm(N, f_bands, a_bands);

% 2. Frequency-Sampling Design

% Frequency points and desired amplitudes

f_grid = [0 fp fs Fs/2] / (Fs/2);

a_grid = [1 1 0 0];

b_fs = fir2(N, f_grid, a_grid);

% Display Numerical Outputs

fprintf('=== EXPERIMENT 5: PARKS-MCCLELLAN vs FREQUENCY SAMPLING ===\n\n');

fprintf('Filter Order (N): %d\n', N);

fprintf('Passband Edge: %d Hz\n', fp);

fprintf('Stopband Edge: %d Hz\n\n', fs);

fprintf('--- PARKS-MCCLELLAN COEFFICIENTS (First 10) ---\n');

disp(b_pm(1:10));

fprintf('--- FREQUENCY-SAMPLING COEFFICIENTS (First 10) ---\n');

disp(b_fs(1:10));

% Frequency Response Calculation

[H_pm, f] = freqz(b_pm, 1, 2048, Fs);

[H_fs, ~] = freqz(b_fs, 1, 2048, Fs);

% Convert Magnitude to dB safely

mag_pm = 20*log10(max(abs(H_pm), 1e-10));

mag_fs = 20*log10(max(abs(H_fs), 1e-10));

% Plotting

figure('Name', 'Parks-McClellan vs Frequency-Sampling');

% Magnitude Response

subplot(2,1,1);

plot(f, mag_pm, 'b', 'LineWidth', 1.5);

hold on;

plot(f, mag_fs, 'r--', 'LineWidth', 1.5);

grid on;

legend('Parks-McClellan (Equiripple)', ...

'Frequency-Sampling', 'Location', 'best');

title(['FIR Filter Magnitude Response Comparison (Order N = ', ...

num2str(N), ')']);

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

xlim([0 Fs/2]);

ylim([-80 10]);

% Phase Response

subplot(2,1,2);

plot(f, unwrap(angle(H_pm))*180/pi, ...

'b', 'LineWidth', 1.5);

hold on;

plot(f, unwrap(angle(H_fs))*180/pi, ...

'r--', 'LineWidth', 1.5);

grid on;

legend('Parks-McClellan', 'Frequency-Sampling', ...

'Location', 'best');

title('Phase Response Comparison');

xlabel('Frequency (Hz)');

ylabel('Phase (degrees)');

xlim([0 Fs/2]);

EXP -8

clc; clear; close all;

%% FIR FILTER PARAMETERS

b = [0.0625 0.2500 0.3750 0.2500 0.0625];

Fs = 1000;

N = 128;

x = [1 zeros(1,N-1)];

n = 0:N-1;

%% 1. DIRECT REALIZATION

yd = filter(b,1,x);

%% 2. CASCADE REALIZATION

z = roots(b);

g = b(1);

% Group conjugate zeros into real first/second-order sections

yc = g*x;

used = false(length(z),1);

tol = 1e-8;

for k = 1:length(z)

if used(k)

continue;

end

if abs(imag(z(k))) < tol

% Real zero: first-order section

section = [1 -real(z(k))];

used(k) = true;

else

% Find the complex-conjugate zero

candidates = find(~used);

candidates(candidates == k) = [];

[~,idx] = min(abs(z(candidates)-conj(z(k))));

kc = candidates(idx);

% Real second-order section

section = real(poly([z(k); z(kc)]));

used(k) = true;

used(kc) = true;

end

yc = filter(section,1,yc);

end

yc = real(yc);

%% 3. SYMMETRY AND LINEAR PHASE

symErr = max(abs(b-fliplr(b)));

D = (length(b)-1)/2;

if symErr < 1e-12

symmetry = 'Symmetric';

else

symmetry = 'Not Symmetric';

end

%% 4. FREQUENCY, PHASE AND GROUP DELAY

[H,w] = freqz(b,1,1024);

phase = unwrap(angle(H));

[gd,wgd] = grpdelay(b,1,1024);

%% 5. NUMERICAL COMPARISON

err = max(abs(yd-yc));

dev = err/max(max(abs(yd)),eps)*100;

%% DISPLAY RESULTS

fprintf('\n============================================\n');

fprintf('       FIR FILTER REALIZATION RESULTS\n');

fprintf('============================================\n');

fprintf('\nFIR Coefficients:\n');

disp(b);

fprintf('Sampling Frequency = %d Hz\n',Fs);

fprintf('Number of Samples = %d\n',N);

fprintf('Coefficient Symmetry = %s\n',symmetry);

fprintf('Symmetry Error = %.3e\n',symErr);

fprintf('Group Delay = %.2f samples\n',D);

fprintf('\nMaximum Error = %.3e\n',err);

fprintf('Percentage Deviation = %.3e %%\n',dev);

%% FIRST 10 IMPULSE RESPONSE VALUES

fprintf('\n============================================\n');

fprintf('       FIRST 10 IMPULSE RESPONSE VALUES\n');

fprintf('============================================\n');

fprintf('%4s %14s %14s\n','n','Direct','Cascade');

for k = 1:10

fprintf('%4d %14.6f %14.6f\n',n(k),yd(k),yc(k));

end

%% CASCADE ZEROS

fprintf('\n============================================\n');

fprintf('          CASCADE REALIZATION ZEROS\n');

fprintf('============================================\n');

disp(z);

%% OUTPUT PARAMETER TABLE

Parameter = {

'Direct realization output'

'Cascade realization output'

'Impulse response'

'Phase response'

'Comparison of realizations'

};

Observation = {

'Direct FIR filtering'

'Cascade of first/second-order sections'

'Equal to FIR coefficient sequence'

'Linear phase'

'Equivalent transfer-function behavior'

};

Value = {

sprintf('[%.4f %.4f %.4f %.4f %.4f]',yd(1:5))

sprintf('[%.4f %.4f %.4f %.4f %.4f]',yc(1:5))

sprintf('[%.4f %.4f %.4f %.4f %.4f]',yd(1:5))

sprintf('Group delay = %.2f samples',D)

sprintf('Max error = %.3e; Deviation = %.3e %%',err,dev)

};

Unit_Note = {

'Amplitude'

'Amplitude'

'h[n]'

'Samples'

'Numerical precision'

};

Output_Table = table(Parameter,Observation,Value,Unit_Note);

fprintf('\n============================================\n');

fprintf('          OUTPUT PARAMETER TABLE\n');

fprintf('============================================\n');

disp(Output_Table);

%% GRAPH 1: DIRECT VS CASCADE IMPULSE RESPONSE

figure('Name','Direct vs Cascade Impulse Response');

stem(n,yd,'b','filled');

hold on;

stem(n,yc,'r--');

grid on;

xlabel('Sample n');

ylabel('Amplitude');

title('Direct and Cascade FIR Impulse Responses');

legend('Direct Realization','Cascade Realization');

%% GRAPH 2: MAGNITUDE RESPONSE

figure('Name','FIR Magnitude Response');

plot(w/pi,abs(H),'LineWidth',1.5);

grid on;

xlabel('Normalized Frequency (\times\pi rad/sample)');

ylabel('|H(e^{j\omega})|');

title('FIR Magnitude Response');

xlim([0 1]);

%% GRAPH 3: PHASE RESPONSE

figure('Name','FIR Phase Response');

plot(w/pi,phase,'LineWidth',1.5);

grid on;

xlabel('Normalized Frequency (\times\pi rad/sample)');

ylabel('Phase (radians)');

title('Unwrapped Phase Response');

xlim([0 1]);

%% GRAPH 4: GROUP DELAY

figure('Name','FIR Group Delay');

plot(wgd/pi,gd,'LineWidth',1.5);

grid on;

xlabel('Normalized Frequency (\times\pi rad/sample)');

ylabel('Group Delay (samples)');

title('Group Delay');

xlim([0 1]);

%% GRAPH 5: NUMERICAL ERROR

figure('Name','Direct vs Cascade Numerical Error');

plot(n,abs(yd-yc),'LineWidth',1.5);

grid on;

xlabel('Sample n');

ylabel('Absolute Error');

title('Direct vs Cascade Numerical Error');

%% FINAL RESULT

fprintf('\n============================================\n');

fprintf('              FINAL RESULT\n');

fprintf('============================================\n');

fprintf('1. FIR filter realized using Direct structure.\n');

fprintf('2. FIR filter realized using Cascade structure.\n');

fprintf('3. Coefficients are %s.\n',symmetry);

fprintf('4. Linear-phase group delay = %.2f samples.\n',D);

fprintf('5. Maximum realization error = %.3e.\n',err);

fprintf('6. Percentage deviation = %.3e %%.\n',dev);

fprintf('7. Direct and Cascade responses are equivalent.\n');